[image: image1.jpg]\WL S¢
< %o

RN
¢
*Aspire P



[image: image6.png]




Premises Management Policy

[image: image7.png]



	Approved by:
	Joanne Kelly
	Date:  4th January 2023

	Reviewed:  
	January 2026

	Next Review:
	January 2028


Contents

31. Aims


32. Guidance


33. Roles and responsibilities


44. Inspection and testing


85. Risk assessments and other checks


96. Monitoring arrangements


97. Links with other policies





1. Aims

Our school aims to ensure that it:

· Manages its buildings and equipment in an efficient, legally compliant way 

· Inspects and tests buildings and equipment regularly, taking into account statutory requirements and best practice recommendations

· Promotes the safety and wellbeing of our staff, pupils, parents and visitors through effective maintenance of buildings and equipment in accordance with the Health and Safety at Work etc. Act 1974
2. Guidance

This document is based on the Department for Education’s guidance on Good Estate Management for Schools extension://efaidnbmnnnibpcajpcglclefindmkaj/https://assets.publishing.service.gov.uk/media/6853da3ed5e318ffbfba2ad6/School_estate_management_standards_amended.pdf
This policy complies with our funding agreement and articles of association.
3. Roles and responsibilities

The Board of Trustees, Head, School Business Manager and Caretaker will ensure this premises management policy is properly implemented, and that tests and inspections are carried out in accordance with this policy. 
The Head and Caretaker are responsible for ensuring relevant risk assessments are conducted and for reporting to the governing board, as required.

The Caretaker is responsible for:

· Inspecting and maintaining the school premises

· Conducting repairs and maintenance

· Being the first point of contact for any issues with the premises

· Conducting and keeping a record of risk assessments and incident logs related to the school premises

· Liaising with the Head and School Business Manager about what actions need to be taken to keep the school premises safe

This list is not intended to be exhaustive.

4. Inspection and testing

We maintain accurate records and details of all statutory tests which are undertaken at our premises. This includes relevant paperwork and certificates. 

All requirements and recommendations highlighted in inspection reports and certificates are reviewed and acted on as necessary. 

As part of the records of completed works, we include the dates when the works were undertaken and the details of the individual or company who completed them, along with their qualifications/certifications and/or experience. 

The table below sets out the issues we inspect, the inspection frequency, and the person responsible for checking each issue and, where appropriate, engaging a suitably qualified person to carry out inspection, testing or maintenance. It covers statutory checks as well as recommended good practice checks from relevant guidance. It is based on the Department for Education’s guidance on good estate management for schools. 
	issue to inspeCT

	frequency
	person responsible

	Portable appliance testing (PAT)

	Annually
Regular visual inspections where PAT is not required.

We will refer to HSE guidance on maintaining portable electric equipment for suggested intervals and types of testing/inspection.
	Caretaker/SBM arranges certified contractor to carry out test.

	Fixed electrical installation tests (including lightning conductors)
	Every five years.  
Inspection and testing always carried out by a competent person.
	Caretaker/SBM arranges certified contractor to undertake test and remedial works.

	Emergency lighting
	Monthly test.
Annual condition test.
	Caretaker/SBM arrange condition test with supplier and any remedial works.

	Gas appliances and fittings
	Routinely, in accordance with manufacturer recommendations (or other professional advice if unavailable). 

Annual safety checks (in line with good practice / required if the premises are used for residential accommodation). 

All work carried out by a Gas Safe Registered engineer.
	Caretaker/SBM arrange for Gas Safe engineer to carry out all checks.

	Air conditioning systems
	Inspections by an energy assessor at regular intervals (not exceeding 5 years). 

Annual certificated inspection to ensure no refrigerant leakage. 

Bi-annual checks and an annual maintenance schedule (in line with good practice).
	Caretaker/SBM arrange service and repairs with supplier.

	Legionella checks on all water systems
	Annual.
	Caretaker/SBM organize with registered contractor.

	Asbestos
	Regular inspections as part of the asbestos register and management plan. 

Reviews of the asbestos register annually. 

Refurbishment and demolition surveys before any refurbishment or demolition work. 
	Caretaker undertakes Asbestos Management training and carries out regular inspections.



	Equipment used for working at height
	Inspected before use, and at suitable intervals appropriate to the environment it’s used in and how it’s used. 

In addition, inspections after anything that may affect the safety or stability of equipment, e.g. adverse weather or accidental damage.
	Caretaker

	Fire detection and alarm systems
	Weekly alarm tests, with a different call point tested each week where applicable. 

Quarterly and annual inspections and tests by a competent person.

Annual fire risk assessment by a competent person also includes the maintenance of fire detection and alarm systems.
	Caretaker carries out weekly alarm tests.
Annual FRA carried out by third party contractor.

	Fire doors
	Regular checks by a competent person.
	Caretaker.  Maintenance by supplier and security contractor.

	Firefighting equipment
	Most equipment – extinguishers, fire blankets, hose reels, fixed systems (such as sprinkler systems) and fire service facilities (such as dry risers and access for emergency vehicles) – inspected annually (by a competent person where required) unless manufacturers’ guidelines suggest differently.
	Caretaker/SBM arrange annual service and remedial works.

	Extraction systems
	Regular removal and cleaning of grease filters and cleaning of ductwork for kitchen extraction systems.

Local exhaust ventilation systems (such as those for working with hazardous substances) examined and tested at least every 14 months by a competent person.

More routine checks also set out in system logbooks. 
	Caretaker arranges works to be carried out.

	Chemical storage


	Inventories are kept up-to-date. 

Risk assessments for the Control of Substances Hazardous to Health (COSHH) are reviewed on a regular basis, plus whenever it’s considered that the original assessment may no longer be valid, or where the circumstances of the work change significantly and may affect employees’ exposure to a hazardous substance (in line with HSE guidance on COSHH assessment).
	Caretaker/Science Teacher

	Playground and gymnasium equipment
	Regular inspections – at least annually, and more regularly where any equipment is used more frequently than normal (e.g. where community use increases how often equipment is used). 

Outdoor fixed play equipment – periodic and annual inspections by a competent person. 
	Caretaker/SBM arrange annual inspection and remedial works with third party contractor

	Tree safety
	As part of risk assessment responsibilities, periodic visual checks for stability are carried out, with more detailed assessments if suspected structural faults or other risks are found.
	SBM arranges 3 yearly tree survey and remedial works by third party contractor.
Caretaker carries out regular visual survey.


5. Risk assessments and other checks

Please refer to our risk assessment policy for information about the school’s approach to risk assessment.

We also make sure further checks are made to confirm the following:
· Correct and up-to-date information is displayed in all notices 

· Compliance with the Construction (Design and Management) Regulations 2015 during construction projects

· Contractors have the necessary qualifications to carry out the specified work 

· Compliance with the Equality Act 2010 when making changes or alterations to a building or the external environment

6. Monitoring arrangements

The application of this policy is monitored by the Caretaker, School Business Manager and Head through, among other things, visual checks of the school site and equipment, and checks of risk assessments. Copies of risk assessments and paperwork relating to any checks are kept in the school office. 

This policy will be reviewed by the Head and School Business Manager every two years.  At every review, the policy will be shared with and approved by the Executive Head and Chair of Trustees. 
7. Links with other policies

This premises management policy is linked to our:

· Health and safety policy

· Risk assessment policy

[image: image2.png]


[image: image3.png]


[image: image4.png]


[image: image5.png]



	Oak Hill School Premises Management Policy
	


Page | 1
	Oak Hill School Premises Management Policy
	


Page | 4

